# ADAS Safety Requirements

ISO 26262 safety analysis for Advanced Driver Assistance System.

# Hazards

HAZ-001: Unintended acceleration

The vehicle accelerates without driver command due to throttle actuator malfunction, potentially leading to collision. ASIL: D

HAZ-002: Loss of braking assist

Electronic braking assist system fails to provide expected braking force during emergency braking. ASIL: D

# Hazardous Events

HE-001: Unintended acceleration on highway

Vehicle accelerates unexpectedly at highway speed (>100 km/h) in moderate traffic. ASIL: D

# Safety Goals

SG-001: Prevent unintended acceleration

The system shall prevent unintended vehicle acceleration exceeding 0.3 m/s² without valid driver throttle input. ASIL: D

SG-002: Ensure minimum braking performance

The system shall ensure braking deceleration of at least 6.5 m/s². ASIL: D

# Functional Safety Requirements

FSR-001: Throttle monitoring and limitation

The system shall independently monitor commanded vs. actual throttle position and cut fuel injection within 100ms if deviation exceeds 5% without driver input.

FSR-003: Brake force monitoring

The system shall monitor brake pressure and activate backup braking within 200ms if deviation >15%.

# Test Cases

TC-001: Throttle monitoring response test

Inject 5% throttle deviation and verify fuel injection cut within 100ms.

TC-003: Brake assist deviation response

Simulate 20% brake force deviation; verify driver alert and backup braking activation within 200ms.